714’s Official Assembly Line Process

for

Building WW2 Piston powered Aircraft

http://home.socal.rr.com/flighttest/

This is the first part of the Assembly Line process. Part 2, when released, will cover the Black Arts and Maneuverability.

This process will help you create historically accurate aircraft for CFS/FS2000 using the actual parameters from the real aircraft. Unfortunately, a few very vocal people insist this cannot be done. The aircraft these people create use wild combinations of distorted data to quickly create an aircraft in an effort to shortcut the aircraft building process. What happens is the developer adds a fudge, this causes another part of the flight envelope to go out of tolerance, so they add another fudge to fix this. Of course, this causes another part of the envelope to go out of tolerance. So, another fudge is added. This fudge process goes on until the developer gets tired of adding fudges. Some developers can last in the fudge process longer than others, so the quality varies from developer to developer and aircraft to aircraft. These aircraft may fly ok within a very narrow range, but are total junk out side of it.

This process proves these individuals are very wrong. When you use the Assembly Line process below, you will create a very accurate flight model much faster than the developers that say it cannot be done. The process below uses physics to determine values, not fudges.

Before we begin, lets define the accuracy we are shooting for. Compare this to the “best” aircraft out there. I challenge you to find anyone’s CFS/FS2000 aircraft that meets ALL of these tolerances (and this includes Microsoft):

+/- 1% Sea Level Speed

+/- 1% Max speed at Altitude

+/- 1% Max Speed Altitude

+/- 1% Stall Speed

+/- 1% Rate of Climb

+/- 1% Service Ceiling

+/- 1% Takeoff Distance

To help everyone, the 714th has gathered lots of information and have created standard parameters for the engines and guns. Make sure you use them if you want accurate flight models. You can find the standard Aircraft Parameters and several utilities at:

http://home.socal.rr.com/flighttest/

You will need Microsoft Excel and the 714 Spreadsheet to design an aircraft using the process. The 714 Spreadsheet contains different pages to do the following:

- Dimensions – Calculates the Planform Drag and other parameters in Section 1204

- Aero – Creates the Section 404 Airfoil AoA vs CL properties and 473 Airfoil Moment tables

- Weight – Calculates the Basic Weight used in Section 1101

- Prop – Creates the Section 511 and 512 parameters for Prop Advance Ratio and Efficiency

- Drag – Allows you to set the correct Drag values

You will also need the following utilities located on the 714’s site:

- The AirEd.Ini file

- The CFS Calculator

Last, you will need Jerry’s AirUpdate program and Flight Test Panel.  You can find all of Jerry’s stuff at:

http://users.erols.com/gwbeckwith/

Before you fly, set the static parameters like weight, moments of inertial, fuel tanks and dimensions. All aircraft need a baseline from which to build, and these are what give your aircraft its basic character.  It is important to fill in the static parameters as accurately as possible.

Assembly Line Check List

| Static and Calculated Parameter Entry |  |
| --- | --- |
| 1) AIR File SetUp |  |
|  | 1) Sections: 401, 404, 430, 473, 511, 512, 546 |
| 2) Spread Sheet Input 1 |  |
|  | 1) 2.1 HowTo, 2.2 Weight, 2.3 Aero, 2.4 Dim, 2.10 Prop |
| Editing the AIR File |  |
| 3) Weight |  |
|  | 1) Section 1101:1Ch – Basic Weight |
|  | 2) Section 1001 – Moments of Inertia. |
| 4) Aerodynamics |  |
|  | 1) 404 – Cl vs AoA Table |
|  | 2) 473 – Wing Moment Coeff vs AoA Table |
|  | 3) 430 - Mach Drag Table |
|  | 4) 401 – Wing Lift vs Mach |
| 5) Dimensions |  |
|  | 1) Section 1204 – Wing |
|  | 2) Section 1205 – Horizontal Stabilizer |
|  | 3) Section 1206 – Vertical Stabilizer |
|  | 4) Section 320 - Control Surfaces |
|  | 5) Section 1004 - Landing Gear |
|  | 6) Section 1005 - Scrape Points |
|  | 7) Section 301 - Fuselage |
| 6) Engine |  |
|  | 1) Section 500 - Engine Piston Engine (FS98/CFS AI) |
|  | 2) Section 505 - Engine CFS/FS2000 Piston Engine |
|  | 3) Max RPM, Section’s 500, 505, 508, 509 |
| 7) Prop |  |
|  | 1) Section 510 - Propeller Parameters |
| 8) Fuel |  |
|  | 1) Section 525-535 Fuel Tanks |
| 9) Set the calculated drag parameters |  |
|  | 1) 1101:50h AoI Drag Adjustment |
|  | 2) 1101:30h - Parasite Drag |
| Close the Air File Parameters Set in Flight |  |
| 10) Set level speeds using the Prop Spreadsheet and Drag Parameter 1101:30h |  |
|  | 1) Prop Input Spreadsheet |
|  | 2) Initial SetUp |
|  | 3) Altitude where Max Speed occurs |
|  | 4) Point 1 = Sea Level Max Speed |
|  | 5) Fine Tune HP using 508 |
|  | 6) Point 2 = Max Speed at Altitude |
|  | 7) Problems: Run-away propeller at altitude |
|  | 8) Point 3 = Rate of Climb |
|  | 9) Takeoff Roll |
|  | 10) Smooth the Prop Points |
| 11) Fine Tuning the Stall Speed with table 473 |  |
|  | 1) Table 473 |
| 12) Fine Tuning the Service Ceiling |  |
|  | 5) Table 509 - Torque 4 |
| The Black Arts |  |
| 13) Maneuverability |  |
|  | 1) Roll Section 1101, Section 518 |
|  | 2) Roll while Pulling g’s Table 546. |
|  | 3) Pulling g’s Section 1101, Section 517 |
|  | 4) Rudder Section 1101, Section 519 |
| 14) Stability Derivitives – The Blackest Art |  |
|  | 6) Section 1101 |

Detailed Assembly Line Instructions

- AIR File Setup

  - Before you begin you must set up your AIR file with default data to correctly work with the Spreadsheet.

  - Use AirEd to first remove and then copy (using AirEd’s copy to clipboard and add from clipboard features) the following Sections from the P-47D located at the Flight Institute:

    - 404 – Cl vs AoA Table

    - 473 – Wing Moment Coeff vs AoA Table

    - 430 - Mach Drag Table

    - 401 – Wing Lift vs Mach

    - 511 – Prop Efficiency

    - 512 – Prop Advance Ratio

    - 546 – Roll while Pulling G’s

- SpreadSheet Input Part 1

  - How To

    - Enter the name of the Aircraft the Spreadsheet is for into Cell B2

    - Hit FILE, SAVE AS and save the spreadsheet using the name of the Aircraft it’s for.

      - Use the format CalcsVx.xx-cc-dddddd.xls where:

        - x.xxx is the version of the spreadsheet being used

        - cc is the Country Code

        - dddddd is the Aircraft Desigination

  - Weight Sheet

    - The Empty Weight for most WW2 fighters does not include the weight of the Guns or other Military accessories.

    - Enter the values to calculate the Basic Weight for the Aircraft.  This is the empty weight, plus the weight of the pilot and any other non-expendable items. For the Pilot and parachute, use 200 lbs.

    - Enter the weight of the guns, you can find this information in the Gun Standard Parameters page at the Flight Institute.  Don’t forget to enter the number of each gun carried.

    - The spreadsheet will calculate the total Basic Weight for the Aircraft from the empty weight plus the weight of the pilot, guns, and any other non-expendable items you entered.  You will enter this value into Section 1101.

    - Enter the Ammo Weight and Fuel to arrive at the Equipped Weight. This weight is only used for performance calculations within the Spreadsheet(s).

  - Aero Sheet

    - Enter the AoA vs CL Aerodynamic characteristics of the airfoil used by the aircraft.

    - You should not need to alter the CM data at this time.

  - Dim Sheet

    - Enter the dimensions for the real aircraft into row 7. Note: You only need to enter data in the green cells only.

    - Aspect Ratio Corrections

      - Mono-Planes:

        - Use a Monk Factor = 1

      - Bi-Planes factor in Munk’s Span Factor using

        - 2 wings Munk Factor=1.1

        - 3 wings Munk Factor=1.2

        - WingSpan is span of the upper wing

        - WingArea is the Area of Both Wings

      - Final Taper Ratio Adjustment u

        - Calculate the Taper Ratio by dividing the Tip Chord by the Root Chord

        - Find the Taper Ratio at the bottom of the graph on the ‘U Calculation’ sheet

        - Move up to the line representing the Aspect Ratio

        - Move horizontally to the left and read the u factor

        - Enter this in the ‘2.4 Dim’ sheet at Cell K7

Editing the AIR File

- Weight

  - Section 1101:1Ch – Basic Weight

    - Enter the Basic Weight Calculated on the 2.2 Weight sheet.

  - Section 1001 – Moments of Inertia.

    - Use the CFS Calculator to determine these values, using the aircraft dimensions and the basic weight calculated on the 2.2 Weight sheet.

- Aerodynamics

  - 404 – Cl vs AoA Table

    - It is important you use the lift coefficient values from the airfoil used on the real aircraft. This affects level speed, climb, and turn performance. Copy the data from the 2.3 Aero sheet and paste it into the file Aero.txt.  Then use AirUpdate to replace the original AIR file data with the new data you pasted into Aero.txt.

    - DO NOT forget to set the Stall Angle parameter in section 1101.

  - 473 – Wing Moment Coeff vs AoA Table

    - It is important you use the same moment coefficient values as the example P-47 located on at the Flight Institute.

  - 430 - Mach Drag Table

    - It is important you use the same drag coefficient values as the example P-47 located at the Flight Institute.

  - 401 – Wing Lift vs Mach

    - It is important you use the same lift coefficient values as the example P-47 located at the Flight Institute.

- Dimensions

  - Section 1204 – Wing

    - Copy the Data from the 2.4 Dim Sheet (Yellow Areas) to Section 1204.

  - Section 1205 – Horizontal Stabilizer

  - Section 1206 – Vertical Stabilizer

  - Section 320 - Control Surfaces

  - Section 1004 - Landing Gear

    - Make sure you turn off the steer-able parameter for tail draggers. You can use differential braking to steer on the ground using the F11 and F12 keys.

    - To set the Gear

      - Set the time of day to Dusk

      - Line the shadows up with the gear.

  - Section 1005 - Scrape Points

  - Section 301 – Fuselage

- Engine

  - Section 500 - Engine Piston Engine (FS98/CFS AI)

    - The HP parameters effects the AI Aircraft’s HP.

  - Section 505 - Engine CFS/FS2000 Piston Engine

    - Make sure you use the parameters from the Standards listed on the Flight Test Site.

  - Max RPM, Section’s 500, 505, 509

    - Make sure to set the Max RPM in all these sections or you will get inconsistent power output

  - Special RPM Settings using 508

    - You must set the Mil and WEP RPM’s in this table to model the correct HP for MIL and WEP HP. Failure to do so will result inconsistent power output

    - Set RPM4 to the MIL RPM

    - Set RPM5 to the WEP RPM

- Prop

  - Section 510 - Propeller Parameters

    - Copy the data from 10 Prop sheet to this section.

    - The most influential parameters are the Prop Diameter and Gear Ratio.

    - Make sure you use the Gear Ratio from the Standards listed on the Flight Test Site.

- Fuel

  - Section 525-535 Fuel Tanks

- Set the drag parameters

  - 1101:50h AoI Drag Adjustment

    - Set to the value recommended by the 9 Drag sheet.

  - 1101:30h - Parasite Drag

    - Set to the value recommended by the 9 Drag sheet.

  - Check Drag Spreadsheet

    - Once the aircraft is flying reasonably well, you can return to the 9 Drag Sheet and review the L/D curve to insure it is in the ballpark.

Close the Air File

# Parameters Set in Flight

- Set level speeds using the Prop Spreadsheet and Drag Parameter 1101:30h

  - Initial SetUp

    - Prop Inputs Sheet

      - Enter the data required into the Prop input sheet

      - Use the MIL RPM to calculate the Prop efficiency

      - Use the Sea Level Speed Calculator to find the Sea Level speed if you do not have it.

      - Adjust Cell F3 until Cell M11 is equal to the max speed at altitude.

      - Set the Runaway Prop factor (Cell H3) to 5 to start.  If necessary, adjust the Runaway Prop factor so that the "Hi Alt" curve on the "Predicted Prop Pitch" graph is less than 65 degrees.  (Cell H6 will display "Increase Runaway Prop Factor!" if cell H3 is unacceptable.)

    - You will be defining 3 points with the 10 Prop sheet. The sheet will estimate these 3 points for you.  If they are not close enough to be useful, you may enter your own values for the prop angles in the 10 Prop sheet as you discover them.

    - Find the Weight Graph (cells C69:M92), set ALL the Weights (cells B74:B84) to the efficiency calculated (for the correct number of Blades, Cells E65-H65) to set the max efficiency factor for the prop. These weight settings will be used to set the drag parameter at section 1101:30h.

    - Use the AirUpdate program to dump just the prop parameters. Leave AirUpdate up since you will need it.

    - Cut the Yellow 512 Section and paste it over the 512 in the Prop.txt file created by AirUpdate.

    - Cut the Yellow 511 Section and paste it over the 511 in the Prop.txt file created by AirUpdate.

    - Save the Prop.txt file

    - Use AirUpdate to update the AIR file. It will update the AIR file and create a backup copy of the AIR file with an extension of “.BAK”.

  - Altitude where Max Speed occurs

    - Note: You’re just setting the altitude where this occurs, not the speed.

    - You will need to set the max speed altitude at this time by adjusting the BoostGain parameter in section 505 of the AIR file. Before doing anything, you must set this to the correct number. Failure to do so will cause problems everywhere.

    - Flying 2000ft above and below the max speed altitude will help you set it.

    - Save the AIR file each time

    - Reload the Aircraft and let the speed stabilize.

    - Repeat until the Max Speed occurs at the max speed altitude.

  - Runaway Prop Check and Max Prop Angle Adjustment

    - After the boost gain is set, check the prop pitch on the flight test gauge when you’re flying at the max speed altitude.  If your aircraft has WEP, enable WEP.

    - The prop pitch should be less than 65 degrees; between 55 and 60 degrees is ideal.

      - If the prop pitch is too high, or the engine is exceeding Max RPM, increase the “Runaway Prop Factor” on the 10 Prop sheet.

      - Copy the corrected 511 and 512 sections from the spreadsheet to the Prop.txt file and use AirUpdate to load them into the AIR file

      - Repeat until the prop angle is between 55 and 60 degrees.

  - Point 1 = Sea Level Max Speed

    - Restart CFS in free flight mode at Sea Level (200 ft over the water is best). Use the autopilot to hold altitude and keep the wings level.

    - This step is only necessary if your aircraft has WEP and the WEP RPM is higher than the MIL RPM:  While the Aircraft accelerates up to speed, use the CTL-F2 and CTL-F3 to reduce the engine speed to the MIL RPM.  (Tip:  You can TEMPORARILY set Max RPM in section 505 to the MIL RPM setting for flight testing.  DON’T forget to set it back to the proper WEP RPM setting when you’re done!)

    - Go into options and set the sim rate to 4x

    - Allow the max speed at SL to stabilize.

    - Adjust the Drag Parameter 1101:30h until the speed at Sea Level is correct. Do not forget to save the AIR file and reload the aircraft in CFS each time you change it.

    - When the airspeed is correct, look on the panel and ENTER THE PROP ANGLE in the 10 Prop sheet.

  - Set HP with Table 508

    - Note: The speed needs to be close to account for a Ram Air effect.  (Don’t forget to reset Max RPM in 505 if you changed it for the SL speed adjustments.)

    - With the max speed set, check the HP.  Do not use WEP.

    - At MIL RPM

      - If the HP is too high, decrease Torque Fac 4 in Section 508

      - If the HP is too low, increase Torque Fac 4 in Section 508

    - At WEP RPM, WEP engaged

      - If the HP is too high, decrease Torque Fac 5 in Section 508

      - If the HP is too low, increase Torque Fac 5 in Section 508

    - Repeat 10.4 Point 1 = Sea Level Max Speed  if necessary

    - When the Sea Speed and HP is correct exit from the AIR file.

  - Point 2 = Max Speed at Altitude

    - In CFS, start the aircraft in Free Flight at its max speed altitude

    - Let the speed stabilize

    - Look on the panel, ENTER THE PROP ANGLE in the 10 Prop sheet, and determine if the speed is too fast or too slow.

    - Go to the spreadsheet and find the Weighting graph at cells B74:B84

    - Find the point closest to the prop angle. RULE: No point can have an efficiency greater than the max efficieny set in 10.1.3. If you come to a point where you need a point greater than the max efficiency to get the speed/Rate of Climb correct there is something wrong. Post a message at the Flight Institute’s Message Board  and ask for help.

      - If the speed is to slow, move the point up

      - If the speed is to fast move the point down

    - Cut the Yellow 511 Section and paste it over the 511 in the Prop.txt file created by AirUpdate.

    - Use AirUpdate to update the AIR file.

    - ALWAYS check to make sure the prop is not exceeding the max RPM. If it does, see below.

    - Go step 10.6.1 and repeat this process until the speed is perfect.

  - Problems: Run-away propeller at altitude

    - Some aircraft, especially those designed to operate at extreme high altitudes, will experience a runaway prop at high altitude. You will need to increase the Runaway Prop Factor at cell H3 until the prop angle at the max speed altitude and speed is below 65 deg. This will fix the prop exceeding the max rpm in level flight.

NOTE:  Run-away propeller at altitude is most likely to occur with WEP.  Checking for and solving this problem should be done with WEP engaged.

  - Point 3 = Rate of Climb

    - Use the Climb speed calculated on cell L62 of the 10 Prop sheet. This is the speed where the best “Rate” of climb occurs. Generally, this will be in the range of 150-190 mph IAS range.  At speeds below and above this, the “Rate” of climb will be less.

    - Use the same technique you used to set the level speeds with the prop. Do not forget to ENTER THE PROP ANGLE in the 10 Prop sheet. This will let you set the correct prop weighting point for the rate of climb.

  - Take-Off Roll

    - You can use the leftmost point to fine tune the takeoff roll. Higher values reduce the distance, lower values increase the distance.

    - The following scenery may help:   http://user.mc.net/~drb/testplane/

  - Smooth the Prop Points

    - Now that you have the 3 points set, you need to smooth all the other points.

    - Do not touch the 3 points you wrote down.

    - When you are done, you should have a nice pointed graph like an upside-down V with the top passing through the Max Sea Level Speed point. If the graph is not similar then there is a problem

- Fine Tuning the Stall Speed with table 473

  - To make it easy on yourself, it is recommended you set up 473 like the example P-47. One point on zero, and the next point at the same angle the wing stalls.

  - Important, when you make a change to 473, you MUST make a corresponding change to 1101:Pitch Moment due to Elevator. If you lower one, you must lower the other.

  - If the stall speed is too high, Lower the one point under the airfoil’s stall (don’t forget to lower the 1101:pitch moment)

Black Art Adjustments

These adjustments are included for completeness to help you get a reasonably well maneuvering aircraft. This area will be addressed in part two of the Assembly Line process.

- Maneuverability

  - Roll

    - Section 1101 has an entire section for roll. For rate adjustment fine-tuning use the Roll Moment and Roll Damping Parameters.

    - Section 518 will allow you to model changes in roll rate vs speed.

  - Roll while Pulling g’s

    - MS messed this up, it should be Roll degradation per AoA. Instead we got Roll Degradation per g

    - As an aircraft fly’s at high AoA’s (more g’s in MS’s table) the aileron is buried in the turbulence of the wing, reducing it’s ability to roll the aircraft. Some aircraft lose all aileron ability to roll the aircraft at high g’s and the pilot must use the rudder to help roll the aircraft. Some aircraft you will have to unload (reduce g) to get them to roll.

    - Table 546 allows you to model this.

  - Pulling g’s

    - The 13 Turn Performance Sheet will give you the approximate turn speeds, radius and rate for the aircraft. Note: this data assumes the pilot is strong enough to pull back on the stick at high speeds and/or the aircraft does not have fabric covered elevators.

    - Section 1101 has an entire section for pitch. For rate adjustment fine-tuning us the Pitch Moment Parameter.

    - Section 517 will allow you to model changes in pitch rate vs speed.

  - Rudder

    - Section 1101 has an entire section for yaw. For rate adjustment fine-tuning use the Yaw Moment Parameter.

    - Section 519 will allow you to model changes in yaw rate vs speed.

- Stability Derivatives – The Blackest Art

  - Section 1101 – Currently work is still preceding on how to set these analytically. So, you’ll need to set these by feel. Keep watch for updates to the Assembly Line Process