The Caproni Ca.135 was an Italian medium bomber designed at Bergamo in Italy by Cesare Pallavicino. It flew for the first time in 1935, and entered service with the Peruvian Air Force in 1937, and with the Regia Aeronautica (Italian Royal Air Force) in January 1938.

The Ca.135 was to be built at Caproni's main Taliedo factory in Milan, which is why the type had a designation in the main Caproni sequence, rather than in the Caproni-Bergamaschi Ca.300 series. However, the project was retained at Ponte San Pietro and the prototype, completed during 1934-35, (a long construction time for the period), was first flown on 1 April. The project chief was Cesare Pallavicino of CAB (Caproni Aereonautica Bergamasca).

Although the new bomber was in the "century series" of Caproni aircraft, it resembled the Caproni Ca.310, with its rounded nose, two engines, low-slung fuselage, and wings with a very long chord. Several versions were fitted with different engines and some had noticeable performance differences.

The prototype was powered by two 836 hp (623 kW) (at 4,000 m) Isotta Fraschini Asso XI RC radial engines initially fitted with two bladed wooden propellers. It had a length of 14.5 m (48 ft), a wingspan of 18.96 m (62.2 ft), and a wing surface of 61.5 m2 (662 sq ft). It weighed 5,606 kg (12,360 lb) empty, and had a 2,875 kg (6,340 lb) useful load. Structurally, it was built of mixed materials, with a stressed-skin forward fuselage, and a wood and fabric-covered steel-tube rear section, the wings being of metal and wood, using fabric and wood as covering. The wings were more than one third of the total length, and had two spars of wooden construction, covered with plywood and metal. The strength coefficient was 7.5. The tail surfaces were made with wood covered with metal and plywood. The fuel system, with two tanks in the inner wings, held a total of 2,200 litres.

The Ca.135's fuselage's shape was quite different than, for example, the Fiat BR.20. If the latter resembled the B-25, the Ca.135, with its low fuselage more resembled the B-26. Its long nose accommodated the bomb-aimer and a front turret (similar to the Piaggio P.108 and later British bombers). The front part of the nose was detachable to assure a quick escape from the aircraft. It also had two doors in the cockpit roof, to allow the pilots to escape in an emergency. The right-hand seat could fold up to assist entry to the nose.

The aircraft was underpowered, with a maximum speed of 363 km/h (226 mph) at 4,500 m (15,000 ft), and a high minimum speed (there were no slats, and maybe not even flaps) of 130 km/h (81 mph). The ceiling was only 6,000 m (20,000 ft) and the endurance, at 70% of throttle was 1,600 km (990 mi). The weight was too high, with total of 8,725 kg (19,240 lb) and not 7,375 kg (16,260 lb) as expected.

The total payload of 2,800 kg (6,200 lb) was shared between the crew (320 or more kg), military equipment (200 kg of weapons, over 100 kg of radio and other equipment), fuel (max. 2,200 litres, 1,500 kg) oil, oxygen and bombs, so with the maximum bombload there was almost no possibility of carrying a full load of fuel (the other Italian bombers were generally capable of 3,300-3,600 kg payload): around 500 kg with the maximum, modest fuel load (the BR.20 and SM.79 had 3,600 litres), or with the maximum bomb load, almost nothing was available with fuel, and the lack of power made take-offs when over-loaded impossible, in fact take-offs were problematic even with a normal load.

Take-off and landing distances were 418 m (1,370 ft) and 430 m (1,400 ft). The range was good enough to assure 2,200 km (1,400 mi) with 550 kg and 1,200 km (750 mi) with 1,200 kg.

The production version was fitted with inline liquid-cooled Asso XI RC 40 engines, giving 900 hp (670 kW) at 4,000 m (13,000 ft), so that aerodynamic drag was reduced, and three-bladed metal propellers, and were theoretically more efficient machines. This new engine gave the aircraft a maximum speed of 400 km/h (250 mph) at 4,000 m., and could climb to 2,000 m in 5.5 minutes, 4,000 m in 12.1 minutes and 5,000 m in 16.9 minutes.

Despite this, the aircraft was still underpowered, which lead to the 1939 Ca.135Mod, fitted with 1,000 hp (750 kW) Piaggio P.XI engines.

Production of the aircraft was initially of only 32 aircraft, of which eight were Ca.135S's, and some were converted into the Ca.135Mod. In 1938 the first Ca.135Bis were built. They were fitted with 1,000 hp Piaggio P.XI RC.40 engines, with Piaggio P.1001 three-blade metal propellers. Length was 17.7 m, wingspan 18.8 m, and wing surface 60 m2. Armament was still only two 12.7 mm guns and one 7.7 mm, but the nose was redesigned to be more aerodynamic. Another 32 aircraft were ordered and built between 1939 and June 1940.

They were not successful aircraft, being heavily criticized by the Italian pilots. Unable to be used operationally, they were sent to flying schools, and then exported to Hungary. The first batch of Ca.135s flown by 11 Wing were phased out by late 1938. Twenty-five were still available at Jesi airfield, though only four were in flying condition, while the others were probably in maintenance for engine replacement. In 1940 there were at least fifteen Ca.135S and Ca.135Mod at the Malpensa flying school, though the poor condition of these aircraft meant that they were scrapped in November 1941. With the scrapping of the first batch and the selling of the second, all sixty-four Ca.135s left the service of the Regia Aeronautica without performing a single operational mission