------- Annex 1 -------------------------------------------

Parameter calculation (1.7 (DOS) / 2.x (Win95) and later)

---------------------------------------------------------

With a very few exceptions it is possible now to do some calculations with the parameters for the commands. Since there is no delimiter character like the "," in other programming languages SCASM needs some help. This is why

you have to use squared brackets "[]" in every arithmetic expression, i.e. [90-3].

Expressions are always evaluated from the left to the right

regardless of the common mathematical rules. The only way to change this is to use nested brackets. Expressions can also contain macro parameter

numbers like this:

[[90 - %3] / 45 ]

Within expressions the following operators are allowed:

+

-

*

/

==      equal (comparison)

!=      not equal

<       less than

<=      less or equal

>       greater than

>=      greater or equal

|       OR (bitwise)

^       exclusive OR

&       AND (bitwise)

<<      shift left

>>      shift right

Note, to perform bitwise operators SCASM converts the involved parts of the expression into long integer format.

Now you can also "cast" the number format if you want. Some examples:

If parameter is labeled as decimal:

12345           normal (default) decimal format

0xAB23          hex format, with "0x" prefix

0b101010101     binary format, with "0b" prefix

AB23            hex number without "0x" prefix

causes error

If parameter is labeled as hex:

AB23            normal (default) hex format,

without prefix

0d12345         decimal format, with "0d" prefix

0b1010101       binary format,

To avoid problems with the current number format recognision routine please use only upper case charaters for hex numbers and use only lower case characters in the "number cast" sequences

"0x." & "0b." & "0d.".

Warning:

--------

The expression evaluator is called for every numeric parameter input. As a side effect arithmetic expressions like "%3-5" will also work.

If the evaluator is called without the "[" at the beginning of the expression, it stops at the first space charater. Therefore I strongly recommend to use the brackets and separate the elements of the expression by spaces. This will also increase the readability of the source code.

Limits:

-------

Since SCASM is a one_pass assembler :Label's in arithmetic expressions cannot be used, if these :Label's are defined in a later source text line. This is because the :Label is still undefined when the expression is evaluated.

NOTE: All calculations are done at compile time, NOT at runtime.

------- Annex 2 -------------------------------------------

General referencepoint memory (1.7 / 2.0)

-----------------------------------------

There is a general referencepoint memory which can be set with the command:

GRP(  <Lat> <Lon> )

Name may be changed

Temporary referencepoint memory

-------------------------------

Every time an Area() command is compiled, the position (Lat/Lon) is stored into a temporary memory. This position is valid until the EndA command is executed. This is not only true for the automatically stored position but also if you explizit define a new GRP(). So a GRP() defined in an Area()

is really a temporary one.

You can use the stored position to calculate a new referencepoint position in every command that requires a Lat/Lon pair input.

There are two options to calculate a new position:

d <delta_Lat/north-south> <delta_Lon/east-west>

r <heading> <distance>

If your new position is 150 meters east and 60 meters south of the stored position you can enter:

RefPoint( abs :Label 1.0  d -60 150 )

and SCASM will calculate the new reference point.

If your new position (i.e. for LandMe) is 400 meters away in

heading 273.4 degrees you can enter:

LandMe ( r 273.4 400    0  90

r  93.4 400    0  27

)

Note: be carefull, the order of the delta distances differs from that in FSOFFS (one of my older programs). This is made to match with the Lat/Lon positions. Do not use this feature for long distances.

------- Annex 3 -------------------------------------------

Variables

---------

SCASM variables are identified by the "$" sign as the first character. They can be used in arithmetic expressions.

Uvar( $name <expression> )

-

This command is used to declare and initialise the

user variable $name.

If this variable is declared inside an Area() it

is asumed to be an temporary variable and is

deleted when the EndA instruction is compiled.

There are also some SCASM internal variables available.

$IC             internal instruction counter,

valid in Area()'s

$PI             The number PI = 3.414 (2.02)

$Section        This is the SCASM internal

section bit variable.

There is only one bit set for

the currently scanned section

(i.e. 0x001 for section 0 = nav

or 0x200 for section 9 = visible

scenery).(2.03)

$Version        The SCASM version number * 100

Variables declared outside of an Area() are valid from the declaration point and can be changed even from inside an Area(). But Variables declared in an Area() are assumed to be local and will be deleted when the EndA command is compiled. User variables are not necessary deleted at the end of a macro!

The user variables have nothing to do with FS5/FS6 internal variables.They are only tools for the experienced programmers.

------- Annex 4 -------------------------------------------

Functions (2.02)

----------------

To improve the parameter calculations there are now some functions available. Please remember, all calculations are done at compile time. Function names are case sensitive.

int[ <expression> ]

This function converts the result of

<expression> into an integer number by

truncating the fractional part.

round[ <expression> ]

This function converts <expression> into an

integer number by doing a normal round

up/down.

sqrt[ <expression> ]

This calculates the square root of

<expression>. The result is still a

floating point value. If needed you

can use:

round[ sqrt[ 100 / 2 ] ]

abs[ <expression> ]

This function makes sure that the result

of <expression> is always a positive number.

sin[ <expression> ]

cos[ <expression> ]

tan[ <expression> ]

asin[ <expression> ]

acos[ <expression> ]

atan[ <expression> ]

atan2[ <expression> <expression> ]

This is the atan2( y / x ) function. Format

corrected in version 2.07

adrpat[ :Label ]

This address patch funktion is a workaround

for the address calculation problem in

Dwx() commands in the case the label is

defined in a later line of the source code.

This function simply adds the current

address to the internal patch table and

returns a 0 value which will be patched

later. (v.2.04)

ipt[ index flag ]

-

version 2.07

index   the wanted point's index number

flag    selects the coordinate element

(x, y, or z) of that point.

-

This function imports point coordinates from

an existing SCASM internal point list.

For example, you have defined an point list

somewhere in an Area(). SCASM holds an

internal copy of this list for the polygon

vector automatic function. This function

gives you access to this list, so you can

copy coordinates to instructions which do

not accept point numbers.

If you want to draw a dotted line with

7 dots from, lets say, point number

5 to 6 of your list, you can write:

DotLine( ipt[5 x] ipt[5 z] ipt[5 y]

ipt[6 x] ipt[6 z] ipt[6 y]   7 )

------- Annex 5 -------------------------------------------

Some color codes to use with the ..Color() commands (hex values). These color codes differ from the numbers you find in the bitmap files.

Colors of variable brightness, depending of the time of

day (F0 colors)

00      black

01      dark gray

02      gray

03      light gray

04      white

05      red

06      green

07      blue

08      orange

09      yellow

0A      brown

0B      beige

0C      orange/brown

0D      green/gray

0E      blue/marine

constant colors, use it for ilumination.

0F      red

10      green

11      blue

12      dark green/blue

13      orange

14      yellow

15      white (high intensity)

16      white/light gray (low intensity)

other colors

21      dark green, replacement color for

some bitmaps ?

2F      brown

------- Annex 6 -------------------------------------------

Some often used FS internal variables (hex)

Most of this variables should not be modified by the user !

Be very carfull when using other FS variables. It is known that the use of some other varables which work in FS5.1 can crash FS6.0

-

282     running bit timer. There is one bit running

from 0000.0000.0000.0001

to 1000.0000.0000.0000

in about 6 seconds. Used for flashlights.

284     crash code (decimal). If you set one of

these values FS will detect a crash.

2       mountain crash

4       crash

8       splash

14      building crash

16      crash with other aircraft (ignored

in FS6)

288     Aircraft is in fuelbox flag. Set it to

1 and your fuel tanks are filled. Aircraft

speed must be nearly zero, otherwise it

seems FS5 ignores this flag.

28A     FS version number, BCD coded.

hex/bcd decimal version

0500    1280    5.0

0510    1296    5.1

0600    1536    6.0/FS95

28C     Time of day code

1       day

2       dusk/dawn

4       night

33B     shortest 3D distance aircraft

-> referencepoint

340     textured ground flag

342     textured buildings flag

346     scenery density code

0       very sparse

1       sparse

2       normal

3       dense

4       very dense

37E     Aircraft delta X(east) coordinate

from Refpoint

382     Aircraft delta Z(alt) coordinate

from RefPoint

386     Aircraft delta Y(north) coordinate

from RefPoint

38A     number of the day (1...365)

38C     year

390     textured water flag (FS5.1 & FS6)

6F8     season code (northern hemisphere)

0 = winter

1 = spring

2 = summer

3 = autum

------- Annex 7 -------------------------------------------

deleted

------- Annex 8 -------------------------------------------

Hex calculation

For some parameters I chose hex values as input. The reason for this is to give you full access to the data records.

To avoid the risk to confuse the new expression evaluator you should only use upper case letters for hex numbers!

Unfortunately hex calculation is not as easy as normal calculation, but I will try to give some help.

Hex conversion table:

decimal hex

0       0

1       1

2       2

3       3

4       4

5       5

6       6

7       7

8       8

9       9

10      A

11      B

12      C

13      D

14      E

15      F

Example:

We want to calculate the flag parameter for

an ILS. We have to add the following values:

80      for the localiser

40      for the glidepath transmitter

01      for the DME (if wanted)

As the '8' and '4' are the same in both

number systems we simply add them in the

decimal system. This yields 12. The

conversion Table gives you the letter 'C'

as hex digit.

---     With this we get the result:

C1

===

If you are not familar with hex arithmetics you

can use the calculator in MS-Windows. You only

need to change it from standard to scientific in

the calculator menu.

------- Annex 9 -------------------------------------------

Information about aditional texture handling options

Sometimes it is pretty difficulty to find the correct X-Y-coordinates for textured polygons. One of the most common examples for this are mountains.

If you are such an lazy scenery designer like me <g> you can try the different features of the bitmap automatic. In this version bitmaps are always aligned to the lower edge or the lowest point of the polygon.

Texture automatic is limited to polygons with a maximum of 16 vertices (vers. 2.29 and later).

T       enables Texture Point calculation automatic

for any ..TexPoly's. SCASM tries to use the

whole texture map, but the aspect ratio is

maintained. Only valid with ..TexPoly's

B       The same as above, but texture aspect ratio

is not maintained. This may result in a

different scaling for the bitmap in x- and

y- direction. Only valid with ..TexPoly's

R       This flag reverses the bitmap in y direction

(up/down). Only valid with ..TexPoly's. This

works only with the T or B flag active.

aditional flags in version 2.04

S scale

maybe changed in next version !!!

With the above flags allone you cannot

control the size of the pixels (scaling).

If this flag is used SCASM expects a

scaling factor for the next parameter. The

scaling factor can be a fractional number

or even an expression.

This scale factor can be calculated as:

(last_usable_pixel

- first_usable_pixel) / longest_dist

If you want to use the whole bitmap and your

distance is 25 meters for your largest

polygon edge, this is simply:

255 / 25 = 10.2

This scale factor is used for X and Y

scaling.

L x1 y1 x2 y2

This flag indicates that 4 aditioanl

parameters follow.

These parameters are bitmap coordinates to

limit the usable bitmap area for this

polygon. SCASM tries to stretch this area

to fit the polygon. If this flag is not

used SCASM asumes

x1 = 0, y1 = 0, x2 = 255, y2 = 255

for the first and the last usable pixel.

This is the whole bitmap.

N       No reset of the S and L values.

Normally the scale factor and the limit

values are resetted everytime a polygon

is computed. If you want to use the

values from a previously defined polygon,

use this flag to tell SCASM not to reset

these values.

example:

...

TexPoly( ats [127/25] L 0 0 127 127   0 1 2 )

TexPoly( atn                          1 2 3 )

...

In this example only a small part of the

bitmap is used and the bitmap scale factor

is calculated to fit a polygon with up

to 25 meters width (x) and height (y).

The second polygon is calculated with the

same scaling and pixel limits since the

previous values are not resetted.

------- Annex 10 ------------------------------------------

Some important BGL section numbers and their hex bitmask values internally used by SCASM. Used in section mask's in Mif() and SCLINK.

0       00001   Nav frequencies (ILS, VOR)

1       00002   Synth tiles (Seed) size 1 to 6

...

6       00002   Synth tiles size 6 (smallest, high

priority / all tiles are internally

handled with the same mask by SCASM)

9       00200   visual scenery

10      00400   object library

11      00800   Airport menu (FS6 and before)

13      02000   ATIS

14      04000   NDB

15      08000   dynamic scenery

16      10000   Markers, LandMe, TimeZone,

elevated surface

------- Annex 11 ------------------------------------------

Macros for use with the Airport 2.xx program.

Macro files for Airport have a standard file extension of ".API" and they are using a standard parameter set. Not all macros are using all parameters. Thats why sometimes dummy values are passed to a macro.

%1      Latitude

%2      Longitude

%3      range (for the Area() command)

%4      scale factor (used in reference points)

%5      heading

%6      user parameter 1 (usually color 1)

%7      user parameter 2 (usually color 2)

%8      user parameter 3 (usually color 3)

%9      user parameter 4 (usually color 4)

%10     v1 value

%11     altitude for reference point

%12     scenery complexity code

%13     "v2="

%14     v2 value

-----------------------------------------------------------