Coyote Avionics

User’s Guide

Coyote Avionics Design

Ike H Slack

05/10/2002

Table of Contents

Introduction									3

S-TEC Autopilot Set								4

S-TEC System 55 Autopilot						5

S-TEC Altitude Selector/Alerter				6

S-TEC Annunciator							8

Garmin Avionics Set								9

GMA 340 Audio Panel							11

GNS 4301 COM1/NAV1/GPS						12

GNS 4302 COM2/NAV2/GPS						13

GNS 5301 COM1/NAV1/GPS with gps overlay			14

GTX 327 Transponder							17

GPS 500 Moving Map Display					18

Garmin VOR2 CDI								19

Repackaged GPS Displays							20

Design Statement								21

Notes										22

Credits										23

References										24

Introduction

The goal of Coyote Avionics Design is to provide modern, high quality avionics for the use of the flightsim community.

The purpose of this document is to provide a central reference for Coyote avionics as they are released.  An updated version of this document will be included with each new release.

S-TEC Autopilot Set

The S-TEC autopilot set simulates the S-TEC System 55 autopilot.  The set consists of autopilot, altitude selector/alerter, and annunciator.

S-TEC System 55 AUTOPILOT

The S-TEC autopilot can be used standalone, or with the Altitude Selector and/or Annunciator.  The autopilot is turned on and off using an external switch.  The AP button that is part of the Mooney_Bravo is perfect for this application.

The HDG, NAV, APR, and REV buttons work just like any other FS2002 autopilot.

A GPS annunciator is provided to indicate that the GPS is providing the NAV source.

ALT button	--	Selecting the ALT button will enter altitude hold mode with holding altitude = current altitude and VS = 0.

VS button	--	Selecting the VS button will enter attitude hold with VS = current vertical rate.

ALT SEL	--	Selecting the hotspot between ALT button and VS button will enter altitude select mode using Altitude Selector altitude and VS.  This simulates the pressing of both the ALT button and the VS button.

VS knob	--	The VS knob will increase/decrease the climb rate in VS mode or altitude select mode.

The vertical speed is displayed in 100 ft increments at the far right of the autopilot window next to the VS annunciator.  A plus (+) value indicates climb and a minus (-) value indicates descent.

Gauge file and name :

cad_StecSys55ap!autopilot

S-TEC Altitude Selector/Alerter

The Altitude Selector/Alerter is an option which is used with the System 55 autopilot.

Display

The leftmost numerical indication is the altitude in 1000 ft increments and the right most numerical indication is the climb/descent rate in 100 ft increments.  For example an altitude reading of 4.0 is 4000 ft and a climb rate of 7 is a climb rate of 700 ft/min.

The annunciators provided are ENT, ALT, ALT SEL, ALR, DH, VS, and BARO.

The Altitude Selector powers up in the ENT BARO mode displaying the current altimeter barometric pressure setting.  The Altimeter knob on the Altimeter can be used to adjust baro, but I usually set baro using 'B'.

BAR button  --	Toggles on and off the BARO display mode in ENT mode.

DTA button	--	Toggles between ENT mode and operate mode.

The first step is usually to select the DTA button which places unit in ALT SEL mode, displaying altitude setting of 0.0.  If ENT is not displayed, unit is in operate mode.

ALT button	--	Toggles between ALT SEL display mode and ALT display mode.

When in ALT display mode the current pressure altitude is displayed.  When in ALT SEL display mode the altitude setting is displayed.

To adjust selected altitude select DTA button to enter ENT ALT SEL mode and adjust inc knob for desired reading.  The knob defaults to 1000 ft increments and clicking the center of the knob will change it to 100 ft increments.  Select ENT button after desired altitude setting is reached to return to operate mode.

VS button	--	Toggles VS display mode on and off.

Select VS button to enter VS display mode and adjust knob for desired climb/descent rate.  In VS display mode each click of the knob increments or decrements the vertical rate setting in 100 ft increments.  VS adjustment is the default and it does not require being in the ENT mode.

NOTE : The VS selection needs to be turned off to adjust altitude or DH settings.  The VS button on the altitude selector toggles the VS display mode on and off, it does not toggle the autopilot VS mode.

ALR button	--	Toggles Altitude Alert mode on and off.

Select the ALR button to turn on the Altitude Alert mode.  If Altitude Alert is turned on the alert sound will occur at 1000 ft before selected altitude and 300 ft before selected altitude.

DH button	--	Toggles Decision Height Alert mode on and off.

To adjust Decision Height (DH), select DTA button to go to ENT mode and then select DH button.  The gauge now indicates decision height setting in 1000 ft increments.  At startup the setting is 0.0.  Adjust the knob for the desired DH.  The  knob will adjust the DH in 100 ft increments.  When DH adjustment is completed select the DTA button to return to operate mode, then toggle DH mode off until ready for approach.  Toggle on DH mode when preparing for landing.  When the aircraft descends to 50' about the DH setting an alert sound will be heard.  As the aircraft continues to descend it will sound an alert again at DH - 50'.

In order for the Altitude Alert and Decision Height Alert system to work the snd gauge must be installed.

Gauge file and name :

cad_StecSys55ap!altpreselector

cad_StecSys55ap!snd		//	required for DH and Alt Alerts

fs2002\Sound\dingdong.wav	//	required for DH and Alt Alerts

S-TEC Annunciator

The annunciator is optional with the S-TEC System 55 autopilot.  Most of the installations I have seen do not use it.  Only the modes supported by FS2002 are implemented.

Gauge file and name :

cad_StecSys55ap!annunciator

Garmin Avionics Set

The Garmin Avionics set consists of six pieces of rack mounted avionics equipment and a full function VOR/LOC/GS/GPS CDI for NAV2.  Any piece can be used alone or in any combination with the other units.  The only exception is that only one   Nav1/Com1 set may be used.  This means that either gns4301 or gns5301 can be used for COM1/NAV1, but not both.

The units are :

gma340      Audio panel

gns4301     COM1/NAV1/GPS

gns4302     COM2/NAV2/GPS

gtx327      Transponder

gns5301     COM1/NAV1/GPS with moving map display

gps500      standalone moving map display

GarminVor2  full function CDI for NAV2

GMA340

GNS430

GTX327

GNS530

GPS500

GMA340 Audio Panel

The audio panel provides full FS2002 audio panel functionality.

Marker Beacon Lights  --  indicates which beacon is being received.

MRKR button         --  Selects Marker Beacon audio output.

COM1 button         --  Selects COM1 audio output.

COM2 button         --  Selects COM2 audio output.

COM1/COM2 button    --  Selects both COM1 and COM2 audio output.

NAV1 button         --  Selects NAV1 cw id audio output.

NAV2 button         --  Selects NAV2 cw id audio output.

DME button          --  Selects DME cw id audio output.

ADF button          --  Selects ADF cw id audio output.

TEST button         --  Cycles thru the Marker Beacon lights (A-O-M) and back to off.

COPILOT button      --  Toggles radio stack window on and off.

Gauge file and name :

cad_garminavionics!gma340

GNS4301 COM1/NAV1/GPS

The GNS4301 provides full COM, NAV and GPS functions.  The GPS portion is a readout of the default FS2002 gps.

Only the standby frequencies may be changed, then they are made active by selecting the appropriate active/standby frequency toggle.

COM1    Both active and standby frequencies are provided.

C<->    Toggles active/standby frequencies.

NAV1    Both active and standby frequencies are provided.

V<->    Toggles active/standby frequencies.

GPS     Default navigation page is provided.  Contains the following information :

CDI     --  Displays +/- 5 NM coarse deviation.

wptid   --  Displays next waypoint id.

DIS     --  Displays distance to next waypoint in NM.

DTK     --  Displays desired track to next waypoint.

BRG     --  Displays bearing to next waypoint.

GS      --  Displays ground speed.

TRK     --  Display current track.

ETE     --  Displays estimated time to next waypoint.

CDI button  	Toggles GPS/NAV autopilot NAV source.  (same function as default gps/nav switch)

GPS CRSR knob	Toggles GPS display on/off.  (same function as default GPS button)

Gauge file and name :

cad_garminavionics!gns4301

GNS4302 COM2/NAV2/GPS

The GNS4302 provides full COM, NAV and GPS functions.  The GPS portion is a readout of the default FS2002 gps.

Only the standby frequencies may be changed, then they are made active by selecting the appropriate active/standby frequency toggle.

COM2    Both active and standby frequencies are provided.

C<->    Toggles active/standby frequencies.

NAV2    Both active and standby frequencies are provided.

V<->    Toggles active/standby frequencies.

GPS     Default navigation page is provided.  Contains the following information :

CDI     --  Displays +/- 5 NM coarse deviation.

wptid   --  Displays next waypoint id.

DIS     --  Displays distance to next waypoint in NM.

DTK     --  Displays desired track to next waypoint.

BRG     --  Displays bearing to next waypoint.

GS      --  Displays ground speed.

TRK     --  Display current track.

ETE     --  Displays estimated time to next waypoint.

CDI button  	Toggles GPS/NAV autopilot NAV source.  (same function as default gps/nav switch)

GPS CRSR knob	Toggles GPS display on/off.  (same function as default GPS button)

Gauge file and name :

cad_garminavionics!gns4302

GNS530 COM1/NAV1/GPS

The GNS530 provides full COM, NAV and GPS functions.  The GPS portion is a readout of the default FS2002 gps.

Only the standby frequencies may be changed, then they are made active by selecting the appropriate active/standby frequency toggle.

COM1    Both active and standby frequencies are provided.

C<->    Toggles active/standby frequencies.

NAV1    Both active and standby frequencies are provided.

V<->    Toggles active/standby frequencies.

VOR     Displays VOR id.

RAD     Displays VOR radial

DIS     Displays DME distance in NM.

GPS     Default navigation page is provided.  Contains the following information :

CDI     --  Displays +/- 5 NM coarse deviation.

wptid   --  Displays next waypoint id.

DIS     --  Displays distance to next waypoint in NM.

DTK     --  Displays desired track to next waypoint.

GS      --  Displays ground speed.

TRK     --  Display current track.

ETE     --  Displays estimated time to next waypoint.

NOTE : Compass rose arc is not implemented in this version.

CDI button	Toggles GPS/NAV autopilot NAV source.  (same function as default gps/nav switch)

GPS CRSR knob	Toggles GPS display on/off.  (same function as default GPS button)

Gauge file and name :

cad_gns530!gns5301

GNSGPS gps overlay

GNS530 with gps overlay

Optional gps map overlay which overlays the nav screen of the GNS530 with the default gps moving map display.  The gps map overlay is a repackaged version of the default FS2002 gps.

Hot spots overlaying RNG buttons --  Arrows pad used to navigate menus and pages.

-D-> button     --  Cycles between Direct To page/Emergency page/Normal page on gps screen.

MENU button     --  Toggles gps Menu page on/off.

CLR button      --  Same function as gps CANCEL button.

ENT button      --  Same function as gps ENTER button.

hot spot below ENT button   --  Cycles display Mode between moving map display, waypoint information display and route information display.

Gauge file and name :

gnsgps!gps

NOTE :  Since FS2002 only supports one GPS nav source toggling NAV source on either NAV1 radio or NAV2 radio has the same effect.  In either case the default GPS becomes the NAV source.

GTX327 TRANSPONDER

The GTX327 Transponder provides modern transponder functions.

VFR button  -- Toggles between previous code and VFR code (1200).

FUNC button -- Toggles between Pressure Altitude display and Flight Time display (flight time display not implemented)

Pressure Altitude is indicated in Flight Level (FL), i.e. altitude = displayed number x100 ft.

SQUAWK CODE -- Squawk Code is entered by clicking above digit to  increase the value or below the digit to decrease it.

Gauge file and name :

cad_garminavionics!gtx327

GPS500 Moving Map Display

This gauge is a replacement for the default gps moving map display.  This unit would typically be used with the gns4301/gns4302 combination.  This is a repackaged version of the default FS2002 gps.

Hot spots overlaying RNG buttons --  Arrows pad used to navigate menus and pages.

-D-> button     --  Cycles between Direct To page/Emergency page/Normal page on gps screen.

MENU button     --  Toggles gps Menu page on/off.

CLR button      --  Same function as gps CANCEL button.

ENT button      --  Same function as gps ENTER button.

GPS CRSR knob   --  Cycles display Mode between moving map display, waypoint information display and route information display.

Gauge file and name :

cad_gps500!gps

Garmin VOR2 CDI

The garminVOR2 CDI is a full function CDI for NAV2.  It provides both VOR/LOC course deviation information and glide slope information.

Gauge file and name :

cad_GarminVor2!gi106b

Repackaged GPS Displays

There are several repackaged GPS displays in our collection.  They are all the default FS2002 GPS with the front panel graphics bitmap changed and the buttons remapped to match the selected GPS.  They all have the same functions as the default GPS.

The following repackaged GPS are currently available :

Apollo MX20 GPS display

ArNav 2000 GPS display

AvMap GPS display

FlightMax GPS display

Garmin GPS 500 GPS display

Garmin GPSMAP 195 GPS display

Garmin GPSMAP 295 GPS display

Garmin GPS Pilot III GPS display

Garmin Vista GPS display

Honeywell GPS display

Most of them were done on request and I am sure the list will continue to grow.

DESIGN STATEMENT :

All of these gauges are original implementations by Coyote Avionics Design except for the repackaged GPS and the garminVOR2 CDI.  The graphics used are either modified versions of vendor product information graphics available on the Internet, scanned images provided by flightsimmers or graphics developed especially for Coyote Avionics Design projects.

The garminVOR2 CDI was implemented using the source code provided by Dai Griffith as part of his excellent tutorial FS2000/CFS SDK Gauge Creation.

The repackaged GPS displays were implemented using information provided by Chuck Dome.

NOTES :

Coyote Avionics are constantly being updated and I will release updates as required.  If you have a project which uses my gauges send me an email and I will send you the latest version.  Please send me an email if anything doesn't work as expected or if there are any questions.

I am always open for suggestions if you have any ideas on improvements that can be made.

CREDITS :

Thanks to Rolf Dieter Bückmann for sharing his source code with others.

Thanks to Chuck Dome who designed gaubmp2 and many of the other utilities we all depend upon, who shared his knowledge and experience when I had questions.

Thanks to Dai Griffith for his great tutorials and for the help he has given myself and other gauge designers.

Thanks to Arne Bartels and all of the other guys out there who support

the various Panel Design forums and are always willing to answer a

fellow programmer's questions.

Thanks to Joerg Banach and Clyde Niles who have been encouraging

me and beta testing my gauges as development progressed.  They both

provided a lot of good ideas which help make my projects better.

REFERENCES :

S-TEC System 55 Pilot Information Manual

Cirrus SR-22 Pilot's Operating Handbook

Garmin GMA 340 Audio Panel Pilot's Guide

Garmin GNS 430 Pilot's Guide and Reference

Garmin GNS 530 Pilot's Guide and Reference

Garmin GPS 500 Pilot's Guide and Reference

Garmin GTX 327 Mode A/C Transponder Pilot's Guide

How To Understand The Inner Workings Of The GPS by Stephen R. Goldsmith is a well written article that has good information on the default gps which also applies to my repackaged gps.  It is a Flightsim.com Howto article which can be found at the following location : http://www.flightsim.com/cgi/kds?$=main/howto/gps-sg.htm

Ike H Slack

Coyote Avionics Design 2002

islack@bellsouth.net